TCP/IP Simulator

Usage Guide  —  Fixture Integration

| Fixture_Company_Server_Simulator Acts as TCP Server — listens for TrueTest connections | Radiant_TrueTest_Client_Simulator Acts as TCP Client — connects out to the fixture server |
| --- | --- |

Contents

1.   Overview

2.   Communication Flow

3.   Packet Frame Format

4.   Getting Started

4.1   Starting the Server  (Fixture Side)

4.2   Starting the Client  (TrueTest Side)

5.   The Packet Builder

6.   Sending a Packet  —  Step by Step

6.1   Command with no data  (e.g. ACK, PING, END)

6.2   Command with ASCII data fields  (e.g. MODELCHG)

6.3   Command with multiple ASCII fields  (e.g. START)

7.   Loading and Sending Data Files

7.1   Fixture → Radiant  |  OTP File  (START command)

7.2   Fixture → Radiant  |  Gamma File  (POCBGAMMA command)

7.3   Radiant → Fixture  |  POCB Data File  (CorrectionData command)

8.   Typical Communication Sequence

9.   Disconnect and Reconnect Behaviour

10.  Command Reference

11.  Debugging Guide

11.1   Opening Files as Code (not Designer)

11.2   Code Structure  —  4-File Layout per Project

11.3   Setting Breakpoints

11.4   Reading the Communication Log

11.5   Common Issues

11.6   Adding Your Own Log Messages

# 1.  Overview

These two standalone Windows applications let you simulate both sides of the TCP/IP communication protocol between Radiant TrueTest and the fixture company's equipment. They are intended as integration reference tools, allowing you to verify your fixture firmware against the exact packet format and command set used by TrueTest, without requiring a live TrueTest installation.

Both applications share an identical UI and packet engine. The only difference is their network role:

- Fixture Company Server Simulator  —  your fixture software should behave like this application. It listens on a port and responds to commands sent by TrueTest.

- Radiant TrueTest Client Simulator  —  simulates what TrueTest sends to your fixture. Use this to drive and test your server implementation.

# 2.  Communication Flow

TrueTest always acts as the TCP client and initiates the connection. The fixture always acts as the TCP server and listens passively on a configured port and IP address.

Connection order:  (1) Start the Server  →  (2) Connect the Client.  The client will keep retrying automatically if the server is not yet listening, so the order is flexible in practice.

# 3.  Packet Frame Format

Every message sent in either direction uses the same fixed frame layout. Understanding this structure is essential for implementing the fixture-side protocol.

| Offset | Size | Field | Description |
| --- | --- | --- | --- |
| 0 – 3 | 4 bytes | Message Size | Total packet length in bytes including this header. Unsigned 32-bit integer, Little-Endian. |
| 4 – 7 | 4 bytes | Checksum | Sum of the 4 message-size bytes (bytes 0–3), stored as a 32-bit integer, Little-Endian. Equal to ((Size>>24)&0xff) + ((Size>>16)&0xff) + ((Size>>8)&0xff) + (Size&0xff). For size=20 (0x14): checksum = 20, stored as [ 14 00 00 00 ]. |
| 8 + | N bytes | ASCII Payload | Space-delimited ASCII string: COMMAND DATA1 DATA2 ... DATAn |
| 8 + N | 1 byte | Null Terminator | 0x00 — marks the end of the ASCII payload. |
| 8+N+1 | M bytes | Raw ByteData | Optional binary payload, always at the very end of the packet after the null terminator. |

NOTE: The ASCII payload always begins with the command keyword (e.g. ACK, START, POCBDATA). Any additional data fields follow, separated by single spaces. The null byte 0x00 terminates the string. Everything after it is treated as raw binary ByteData.

Example wire bytes for  START SN001  (no ByteData, size = 4+4+11+1 = 20 = 0x14):

[ 14 00 00 00 ]  [ 14 00 00 00 ]  [ 53 54 41 52 54 20 53 4E 30 30 31 ]  [ 00 ]

size LE              checksum LE       S  T  A  R  T     S  N  0  0  1      null

# 4.  Getting Started

## 4.1  Starting the Server  (Fixture Side)

- Open  Fixture_Company_Server_Simulator.exe.

- In the Listen IP field, enter the local IP address of the fixture machine (default: 127.0.0.1 for local testing).

- In the Port field, enter the listening port number (default: 5000).

- Click Start Listening.  The status changes to Listening… and the Disconnect button becomes enabled. The application now waits for an incoming connection.

- When TrueTest (or the Client Simulator) connects, the status changes to Client Connected and all received packets appear in the Communication Log on the right.

TIP: After a client disconnects, the server automatically returns to Listening state without requiring a manual restart. It is immediately ready to accept the next connection.

## 4.2  Starting the Client  (TrueTest Side)

- Open  Radiant_TrueTest_Client_Simulator.exe.

- Enter the Server IP (the IP address where the fixture server is running).

- Enter the same port number used by the server.

- Click Connect.  The client immediately begins attempting to connect. If the server is not yet listening it will automatically retry every 2 seconds, logging each attempt:

Attempt 1  —  connecting to 127.0.0.1:5000 ...

Attempt 1 failed  (No connection could be made)  —  retrying in 2 s...

Attempt 3  —  connecting to 127.0.0.1:5000 ...

Connected to fixture server at 127.0.0.1:5000  (after 3 attempts)

- Click Disconnect at any time to cancel retrying or close an active connection.

TIP: When the server disconnects first, the client automatically resumes retrying until the server starts listening again. No manual action is required on the client side.

# 5.  The Packet Builder

Both applications share an identical Packet Builder panel on the left side of the window. Use it to manually compose and send any packet to the connected peer.

| UI Element | Purpose |
| --- | --- |
| Command dropdown | Select the command to send. Contains all 52 protocol commands. The selected value becomes the first token of the ASCII payload. |
| ASCII Data Items list | An ordered list of additional ASCII string fields. Each entry is appended after the command, separated by a space. Use the text box and Add button to insert items, Remove to delete the selected item, and Clear All to reset. |
| File Format selector | Specifies how the loaded file is decoded into bytes before sending. Must be set correctly before clicking Load from File. See Section 7 for details per file type. |
| Load from File... | Opens a file picker. The file is read and decoded according to the File Format setting. The result is stored as the ByteData payload for the next packet. |
| Clear ByteData | Removes the currently loaded ByteData. The next packet will be sent without a binary payload. |
| SEND PACKET (green button) | Assembles and transmits the packet: command string, space, data fields, null terminator, then optional ByteData. Only enabled while connected. |

# 6.  Sending a Packet  —  Step by Step

## 6.1  Command with no data  (e.g. ACK, PING, END)

- Select the command from the Command dropdown (e.g. ACK).

- Ensure the ASCII Data Items list is empty. Click Clear All if needed.

- Ensure no ByteData is loaded. Click Clear ByteData if needed.

- Click SEND PACKET.

Example wire bytes for  ACK  (no ByteData, size = 4+4+3+1 = 12 = 0x0C):

[ 0C 00 00 00 ]  [ 0C 00 00 00 ]  [ 41 43 4B ]  [ 00 ]

size LE            checksum LE    A  C  K     null

## 6.2  Command with ASCII data fields  (e.g. MODELCHG, CHGPTN)

- Select the command from the dropdown (e.g. MODELCHG).

- Type each data value in the text box and click Add. Repeat for each field.

Example  —  sending model name D99Orbit:

Command:      MODELCHG

Data field 1: D99Orbit

- Click SEND PACKET.

Example wire bytes for  MODELCHG D99Orbit  (no ByteData, size = 4+4+17+1 = 26 = 0x1A):

[ 1A 00 00 00 ]  [ 1A 00 00 00 ]  [ 4D 4F 44 45 4C 43 48 47 20 44 39 39 4F 72 62 69 74 ]  [ 00 ]

size LE            checksum LE    M  O  D  E  L  C  H  G     D  9  9  O  r  b  i  t     null

## 6.3  Command with multiple ASCII fields  (e.g. CHGRGB with R, G, B values)

Each data field you add to the list becomes one space-separated token in the payload after the command keyword. There is no limit to the number of fields.

- Select the command from the dropdown (e.g. CHGRGB).

- Type each data value in the text box and click Add. Add one field at a time.

Example  —  CHGRGB with separate R, G, B values:

Command:      CHGRGB

Data field 1: 120

Data field 2: 230

Data field 3: 255

- Click SEND PACKET.

Example wire bytes for  CHGRGB 120 230 255  (no ByteData, size = 4+4+18+1 = 27 = 0x1B):

[ 1B 00 00 00 ]  [ 1B 00 00 00 ]  [ 43 48 47 52 47 42 20 31 32 30 20 32 33 30 20 32 35 35 ]  [ 00 ]

size LE            checksum LE    C  H  G  R  G  B     1  2  0     2  3  0     2  5  5     null

NOTE: Data fields are appended in the exact order they appear in the list. Use the Remove button to delete a selected item, or Clear All to start over.

# 7.  Loading and Sending Data Files

Two types of hex-text file format are used. The File Format dropdown in the Packet Builder has exactly two options. Always select the correct one before clicking Load from File, or the bytes will be decoded incorrectly.

| File Format Option | When to use it |
| --- | --- |
| Hex text, one byte per line | Gamma File.hex and POCBDATA.hex — files where each line contains exactly one two-character hex byte value. |
| Hex text, space-separated | OTP File Type 1.txt and OTP File Type 2.txt — files where hex values are comma-separated or space-separated, prefixed with 0x. |

## 7.1  Fixture → Radiant  |  OTP File  (START command)

The fixture sends an OTP binary payload to TrueTest as part of the START command. Two OTP file formats are provided as samples — both use the same File Format option.

| File | Content example | Notes |
| --- | --- | --- |
| OTP File Type 1.txt | 0x6900,0xF0,0x03,0x12,0x0B | Single line, comma-separated, 0x-prefixed. The 0x6900 header token is automatically stripped. |
| OTP File Type 2.txt | 0x00 0x3A 0x3E 0x42 ... | Multiple lines, space-separated, 0x-prefixed values. |

Steps to send an OTP file from the Server Simulator:

- In the Packet Builder, set File Format to:  Hex text, space-separated (OTP Type 1 & 2)

- Click Load from File… and select OTP File Type 1.txt  or  OTP File Type 2.txt

- The byte count is automatically added as the last item in the ASCII Data Items list:

ASCII Data Items:  [ 5 ]

- Add any other required fields before it — at minimum the serial number.  Example:

ASCII Data Items:  [ SN12345678 ]  [ 5 ]

- Select command:  Start

- Click SEND PACKET.  The wire payload will be:

"START SN12345678 5"  +  0x00  +  [5 bytes of OTP binary data]

NOTE: The byte count is always visible in the ASCII Data Items list after loading a file, and is removed automatically if you click Clear ByteData. You can still reorder or edit items in the list if needed.

## 7.2  Fixture → Radiant  |  Gamma File  (POCBGAMMA command)

The fixture sends gamma correction data to TrueTest as a binary payload attached to the POCBGAMMA command. The sample file is Gamma File.hex.

File format — one hex byte value per line with CRLF line endings:

A3

4F

B2

...

Steps to send the Gamma file from the Server Simulator:

- In the Packet Builder, set File Format to:  Hex text, one byte per line (Gamma / POCBDATA)

- Click Load from File… and select Gamma File.hex

- The byte count is automatically added as the last item in the ASCII Data Items list:

ASCII Data Items:  [ 714752 ]

- Select command:  POCBGAMMA

- Click SEND PACKET.  The wire payload will be:

"POCBGAMMA 714752"  +  0x00  +  [714752 bytes of Gamma binary data]

## 7.3  Radiant → Fixture  |  POCB Data File  (CorrectionData command)

TrueTest sends POCB pixel correction data to the fixture as a binary payload attached to the CorrectionData (POCBDATA) command. The sample file is POCBDATA.hex.

File format — one hex byte value per line with LF line endings:

A3

4F

B2

...

Steps to send the POCB file from the Client Simulator:

- In the Packet Builder, set File Format to:  Hex text, one byte per line (Gamma / POCBDATA)

- Click Load from File… and select POCBDATA.hex

- The byte count is automatically added as the last item in the ASCII Data Items list:

ASCII Data Items:  [ 3993600 ]

- Select command:  CorrectionData

- Click SEND PACKET.  The wire payload will be:

"POCBDATA 3993600"  +  0x00  +  [3993600 bytes of POCB binary data]

TIP: After receiving POCBDATA, the fixture typically responds with ACK, then sends either END (sequence complete) or CHGRCB (switch to RCB mode). Use the Packet Builder to send the appropriate response.

# 8.  Typical Communication Sequence

The following is a representative sequence of commands exchanged during a standard panel calibration run. The fixture always initiates the cycle with START. Every command expects an ACK from the opposite side before the next command is sent.

| # | Sender | Command | Notes |
| --- | --- | --- | --- |
| 1 | Server (Fixture) | MODELCHG D99Orbit | Fixture reports the panel model name first so TrueTest can prepare for the correct panel type. |
| 2 | Client (TrueTest) | ACK | TrueTest acknowledges model info. |
| 3 | Server (Fixture) | START SN12345678 [byte count] | Fixture initiates — new panel is ready. Carries serial number. If OTP data is attached, byte count auto-added; binary hex file as ByteData. |
| 4 | Client (TrueTest) | ACK | TrueTest acknowledges START. |
| 5 | Client (TrueTest) | CHGPTN 1 | TrueTest requests pattern change to index 1. |
| 6 | Server (Fixture) | ACK | Fixture acknowledges pattern request. |
| 7 | Client (TrueTest) | CHGPTNDONE | TrueTest confirms the pattern change is done. |
| 8 | Server (Fixture) | ACK | Fixture acknowledges. |
| 9 | Client (TrueTest) | POCBDATA 3993600 | TrueTest sends computed POCB correction data. Byte count auto-added; binary hex file as ByteData. |
| 10 | Server (Fixture) | ACK | Fixture acknowledges receipt of correction data. |
| 11 | Server (Fixture) | END or CHGRCB | Fixture signals sequence end (END) or requests TrueTest switch to RCB mode (CHGRCB). |
| 12 | Client (TrueTest) | ACK | TrueTest acknowledges. |
| 13 | Client (TrueTest) | RESULT OK | TrueTest reports the final panel result. |
| 14 | Server (Fixture) | ACK | Fixture acknowledges result. |

# 9.  Disconnect and Reconnect Behaviour

The simulators recover automatically from disconnections so you can repeatedly test connect / disconnect cycles without restarting either application.

| Event | Server behaviour | Client behaviour |
| --- | --- | --- |
| Client disconnects first | Automatically returns to Listening state. TcpListener keeps its port binding — no restart needed. | — |
| Server disconnects first | — | Automatically resumes retrying every 2 seconds until the server starts listening again. |
| User clicks Disconnect (server) | Fully stops the TcpListener. User must click Start Listening again to accept new connections. | — |
| User clicks Disconnect (client) | — | Immediately stops the retry loop. User must click Connect to start again. |

# 10.  Command Reference

All 52 protocol commands are listed below. The Wire String column shows the exact ASCII token written into the packet payload. The Direction column shows the typical originator, though both applications can send or receive any command.

| Enum Name | Wire String | Direction | Description |
| --- | --- | --- | --- |
| Start | START | Server → Client | Fixture initiates a new panel test cycle. Carries serial number and OTP binary data. |
| Ends | END | Server → Client | Fixture signals the current test sequence has ended. |
| ModelChange | MODELCHG | Server → Client | Fixture reports the panel model name to TrueTest (e.g. D99, Orbit). |
| ChangePattern | CHGPTN | Client → Server | TrueTest requests the fixture to change the display test pattern. |
| CorrectionData | POCBDATA | Client → Server | TrueTest delivers computed POCB pixel correction data. Binary hex file as ByteData. |
| Result | RESULT | Client → Server | TrueTest sends the final pass / fail result for the panel. |
| PGOn | PGON | Client → Server | TrueTest turns the pattern generator ON. |
| PGOff | PGOFF | Client → Server | TrueTest turns the pattern generator OFF. |
| ACK | ACK | Either direction | Acknowledge the previous packet — no error. |
| NAK | NAK | Either direction | Negative-acknowledge — error or reject. |
| PING | PING | Either direction | Keep-alive / heartbeat. No data payload. |
| ChgPtnDone | CHGPTNDONE | Client → Server | TrueTest confirms the requested pattern change is complete. |
| ModeChangeLPM | CHGLPM | Client → Server | TrueTest requests the fixture switch to Low Power Mode. |
| CHGRCB | CHGRCB | Server → Client | Fixture instructs TrueTest to switch to RCB correction mode. |
| POCBGAMMA | POCBGAMMA | Server → Client | Fixture sends gamma correction data to TrueTest. Binary hex file as ByteData. |
| CAMALARM | CAMALARM | Client → Server | TrueTest reports a camera alarm condition to the fixture. |
| GATEDATA | GATEDATA | Server → Client | Fixture delivers gate-driver data to TrueTest. |
| FFCSTART | FFCSTART | Server → Client | Fixture starts flat-field correction. |
| FFCDONE | FFCDONE | Client → Server | TrueTest confirms flat-field correction is complete. |
| POCBPATH | POCBPATH | Client → Server | TrueTest provides the POCB correction file path(s) to the fixture. |
| STAINSTART | STAINSTART | Server → Client | Fixture starts stain measurement. |
| STAINDONE | STAINDONE | Client → Server | TrueTest confirms stain measurement is complete. |
| CHGBND | CHGBND | Client → Server | TrueTest changes band / frequency setting. |
| FTPUPLOAD | FTPUPLOAD | Server → Client | Fixture sends FTP upload info (host, user, password, etc.) to TrueTest. |
| GETVER | GETVER | Server → Client | Fixture requests TrueTest to report its software version. |
| DGMA | DGMA | Server → Client | Fixture sends digital-gamma data to TrueTest. Binary hex file as ByteData. |
| PUCVER | PUCVER | Client → Server | TrueTest reports the PUC version to the fixture. PUC is the customer-side POCB processing program. |
| GETSEQUENCE | GETSEQUENCE | Server → Client | Fixture requests TrueTest to report the current test sequence name. |
| HOTCOLDPIXEL | HOTCOLDPIXEL | Client → Server | TrueTest sends hot / cold pixel detection result. |
| INTENSITY | INTENSITY | Client → Server | TrueTest sends intensity measurement result. |
| MOIRE | MOIRE | Client → Server | TrueTest sends Moire pattern detection result. |
| PWMSIGNAL | PWMSIGNAL | Client → Server | TrueTest sends PWM signal measurement result. |
| AFTERSTART | AFTERSTART | Server → Client | Fixture starts after-image measurement. |
| AFTERDONE | AFTERDONE | Client → Server | TrueTest confirms after-image measurement is complete. |
| DARKPOINTSTART | DARKPOINTSTART | Server → Client | Fixture starts dark-point measurement. |
| DARKPOINTDONE | DARKPOINTDONE | Client → Server | TrueTest confirms dark-point measurement is complete. |
| DARKPOINTDATA | DARKPOINTDATA | Client → Server | TrueTest sends dark-point measurement data. |
| TEMPLATE | TEMPLATE | Server → Client | Fixture notifies TrueTest to start and handle a template-related flow. |
| CHGPUC | CHGPUC | Client → Server | TrueTest requests the fixture to change the POCB (PUC) program status — ON or OFF. |
| BMPDATA | BMPDATA | Client → Server | TrueTest delivers bitmap image data. |
| SEQCHECK | SEQCHECK | Server → Client | Fixture requests TrueTest to verify the current test sequence. |
| SEQCHECKDONE | SEQCHECKDONE | Client → Server | TrueTest confirms sequence check is complete. |
| TABLETRESULT | TABLETRESULT | Client → Server | TrueTest sends tablet-mode test result. |
| RESTART | RESTART | Server → Client | Fixture triggers a restart-related function in TrueTest. |
| TIME | TIME | Server → Client | Fixture requests TrueTest to synchronise time with the fixture. |
| CAMSTOP | CAMSTOP | Server → Client | Fixture requests TrueTest to stop camera capture. |
| VNTTCheck | VNTTCheck | Client → Server | TrueTest reports VNTT calibration data to the fixture. |
| VNTTset | VNTTset | Server → Client | Fixture requests TrueTest to update VNTT calibration files. |
| CHGRGB | CHGRGB | Client → Server | TrueTest requests the fixture to change display pattern to a specific RGB colour. |
| VNTTCOPY | VNTTCOPY | Server → Client | Fixture requests TrueTest to copy VNTT configuration files. |

# 11.  Debugging Guide

Both simulators are built entirely in code with no Visual Studio Designer file. This section explains how to navigate and debug the code effectively.

## 11.1  Opening Files as Code (not Designer)

Because the form UI is built programmatically, double-clicking a .vb file in Solution Explorer may open the blank Designer view. To open the code editor:

- Right-click the file in Solution Explorer (UI.vb, EventHandlers.vb, Server.vb / Client.vb, or Protocol.vb).

- Choose View Code  (or press F7).

NOTE: The .vbproj files are already configured with <SubType>Code</SubType> on every file, which should make double-click open the code editor directly. If the Designer still appears, use the right-click method above.

## 11.2  Code Structure  —  4-File Layout per Project

Each project is split into two Partial Class files that compile into one class. There is no Designer.vb.

| File | Contains | Key things to look at |
| --- | --- | --- |
| UI.vb | BuildUI() Control declarations SetConnectedState() SetListeningState() or SetRetryingState() | Start here when debugging UI layout, missing controls, or status indicator problems. |
| EventHandlers.vb | OnConnectClick / OnListenClick OnSendClick OnLoadByteDataClick OnClearByteDataClick CommandItem class mByteData / mAutoByteCount | Start here when debugging button behaviour, file loading errors, or the wrong command being sent. |
| Server.vb or Client.vb | StartListening / ConnectToServer DoConnect (retry loop) ReceiveLoop RestartListening / RestartRetrying SendPacket Log() | Start here when debugging connection issues, packet parsing, reconnect behaviour, or send failures. |
| Protocol.vb | CommandId enum (52 commands) Endness enum TcpPacket class ParseCommandString() GetCommandString() Send() | Start here when debugging packet wire format, a command that shows as Unknown, or checksum / size calculations. |

## 11.3  Setting Breakpoints

Open the relevant file in the code editor and click in the left margin next to any line to set a breakpoint. The most useful locations are:

| Method / Location | What it tells you |
| --- | --- |
| ReceiveLoop — bytesRead = 0 line | Triggers when client disconnects. Check the mConnected and mConnectionId state. |
| ReceiveLoop — after pkt = New TcpPacket(…) | Inspect pkt.Command, pkt.Data, pkt.ByteDataLength to see exactly what was parsed from the raw buffer. |
| ParseCommandString — Case Else return | Triggers when an unrecognised command string arrives. Inspect the s parameter to see what string the receiver got. |
| SendPacket — pkt.Send(mStream) | Confirm what is actually being serialised and sent. Step into pkt.Send() to watch the packet assembly. |
| OnLoadByteDataClick — mByteData = result.ToArray() | Inspect mByteData.Length to confirm the correct number of bytes were decoded from the file. |
| OnSendClick — SendPacket(cmd, dataList, mByteData) | Check cmd, dataList contents, and mByteData before the packet is built. |

## 11.4  Reading the Communication Log

Every sent and received packet is logged with a timestamp and both the enum name and wire string:

[2026/04/16 10:04:09.266]  <<< SENT  [ChangePattern  /  wire: CHGPTN]    Data: 0

[2026/04/16 10:04:09.310]  >>> RECEIVED  [ACK  /  wire: ACK]

| Log token | Meaning |
| --- | --- |
| <<< SENT | Packet successfully written to the network stream. |
| >>> RECEIVED | Packet successfully read and parsed from the network stream. |
| [ChangePattern / wire: CHGPTN] | Enum name (used in code) followed by the actual ASCII string sent over TCP. |
| Data: 0 | ASCII data fields extracted from the payload, shown pipe-separated if multiple. |
| + N bytes of raw ByteData | Number of binary bytes that followed the null terminator in this packet. |
| SEND FAILED | The stream.Write() threw an exception. Check if the connection is still active. |
| Incorrect file type | File format validation failed — the selected File Format does not match the file content. |

## 11.5  Common Issues

| Symptom | What to check |
| --- | --- |
| Client keeps retrying, never connects | Confirm the Server is running and "Listening" before clicking Connect. Check IP and Port match on both sides. Check Windows Firewall is not blocking the port. |
| Connected but command shows [Unknown] | Set a breakpoint in ParseCommandString at the Case Else line. Inspect the s parameter — the incoming string does not match any Case. Check for trailing spaces or different casing. |
| Byte count in Data list does not match file | Check the File Format selection matches the actual file format. Load the file again after correcting the format. The byte count in the list updates automatically on each load. |
| SEND FAILED in log | The connection dropped between clicking Connect and clicking Send. Check the status indicator — reconnect if needed. |
| Form appears but all controls are missing | BuildUI() was not called in the constructor. Check that Sub New() calls BuildUI() in UI.vb. |
| Load event SplitterDistance throws exception | The form's minimum size is smaller than Panel1MinSize + Panel2MinSize. Increase Me.MinimumSize or reduce the panel minimum sizes in the Load handler. |

## 11.6  Adding Your Own Log Messages

The Log() method is thread-safe and available anywhere in the Partial Class. Call it from any method to trace custom information:

Log("My debug message: " & someVariable.ToString())

Log() automatically prepends a timestamp and appends the entry to the Communication Log listbox. It is safe to call from background threads (ReceiveLoop, ListenForClient, DoConnect) — it uses BeginInvoke internally to marshal to the UI thread.

End of document  —  Radiant Vision Systems / Fixture Integration Reference